8a.Simulation of EMI Sources and Interference Analysis using MATLAB

OVERALL MATLAB CODE:

clc;

clear;

close all;

%% Objective 1: Generate Desired Signal and EMI Sources

fs = 10000; % Sampling Frequency

t = 0:1/fs:0.01; % Time Vector

% Desired Communication Signal

A1 = 1;

f1 = 1000;

signal = A1*sin(2*pi*f1*t);

% EMI Source 1 (Sinusoidal Interference)

Aemi = 0.5;

femi = 2500;

emi_sine = Aemi*sin(2*pi*femi*t);

% EMI Source 2 (Broadband Noise)

emi_noise = 0.3*randn(size(t));

% EMI Source 3 (Impulse Noise)

emi_impulse = zeros(size(t));

emi_impulse(20:20:end) = 1;

% Total EMI

emi_total = emi_sine + emi_noise + emi_impulse;

% Received Signal

received = signal + emi_total;

%% Objective 2: Time Domain and Frequency Domain Analysis

N = length(received);

f = (-N/2:N/2-1)*(fs/N);

Signal_FFT = abs(fftshift(fft(signal)))/N;

EMI_FFT = abs(fftshift(fft(emi_total)))/N;

Received_FFT = abs(fftshift(fft(received)))/N;

%% Objective 3: SNR Analysis and EMI Mitigation

% Signal Power

Ps = mean(signal.^2);

% Noise Power Before Filtering

Pn_before = mean(emi_total.^2);

% SNR Before Filtering

SNR_before = 10*log10(Ps/Pn_before);

% Low Pass Filter

filtered = lowpass(received,1500,fs);

% Noise Power After Filtering

Pn_after = mean((filtered - signal).^2);

% SNR After Filtering

SNR_after = 10*log10(Ps/Pn_after);

%% Display Results

fprintf('\n---------------------------------\n');

fprintf(' EMI ANALYSIS RESULTS\n');

fprintf('---------------------------------\n');

fprintf('Signal Frequency = %d Hz\n',f1);

fprintf('EMI Frequency = %d Hz\n',femi);

fprintf('\nSignal Power = %.4f W\n',Ps);

fprintf('Noise Power Before = %.4f W\n',Pn_before);

fprintf('Noise Power After = %.4f W\n',Pn_after);

fprintf('\nSNR Before Filtering = %.2f dB\n',SNR_before);

fprintf('SNR After Filtering = %.2f dB\n',SNR_after);

fprintf('SNR Improvement = %.2f dB\n', ...

(SNR_after-SNR_before));

%% Plot Results

figure('Name','EMI Simulation and Analysis',...

'NumberTitle','off');

subplot(4,2,1)

plot(t,signal)

grid on

title('Desired Signal')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,2)

plot(t,emi_sine)

grid on

title('Sinusoidal EMI')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,3)

plot(t,emi_noise)

grid on

title('Broadband EMI Noise')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,4)

stem(t,emi_impulse)

grid on

title('Impulse EMI')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,5)

plot(t,received)

grid on

title('Received Signal with EMI')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,6)

plot(f,Received_FFT)

grid on

title('Frequency Spectrum of EMI Signal')

xlabel('Frequency (Hz)')

ylabel('Magnitude')

subplot(4,2,7)

plot(t,filtered)

grid on

title('Filtered Signal')

xlabel('Time (s)')

ylabel('Amplitude')

subplot(4,2,8)

bar([SNR_before SNR_after])

grid on

title('SNR Comparison')

ylabel('SNR (dB)')

set(gca,'XTickLabel',{'Before','After'})

sgtitle('Simulation of EMI Sources and Interference Analysis using MATLAB')

OUTPUT