8b.Shielding Effectiveness and EMC Compliance Analysis using MATLAB

OVERALL MATLAB CODE:

clc;

clear;

close all;

%% Objective 1 : Shielding Effectiveness Simulation

f = logspace(6,9,1000); % 1 MHz to 1 GHz

mu0 = 4*pi*1e-7;

sigma = 5.8e7; % Copper conductivity

t = 1e-3; % Shield thickness

omega = 2*pi*f;

delta = sqrt(2./(omega*mu0*sigma));

A = 8.686*(t./delta); % Absorption Loss (dB)

%% Objective 2 : Reflection and Multiple Reflection Loss

sigma_r = 1;

mu_r = 1;

R = 168 - 10*log10((sigma_r)./(mu_r*f));

B = 5; % Approximate Multiple Reflection Loss

SE = R + A + B;

%% Objective 3 : EMC Compliance Analysis

EMC_Limit = 50*ones(size(f));

Measured_Emission = ...

40 + 5*sin(2*pi*log10(f)/3);

Margin = EMC_Limit - Measured_Emission;

Compliance = Margin > 0;

%% Display Results

fprintf('\nSHIELDING EFFECTIVENESS RESULTS\n');

fprintf('---------------------------------\n');

fprintf('Maximum SE = %.2f dB\n',max(SE));

fprintf('Minimum SE = %.2f dB\n',min(SE));

if all(Compliance)

fprintf('System is EMC COMPLIANT\n');

else

fprintf('System is NOT EMC COMPLIANT\n');

end

%% Plot Results

figure('Name',...

'Shielding Effectiveness and EMC Compliance',...

'NumberTitle','off');

subplot(2,2,1)

semilogx(f,A,'LineWidth',2)

grid on

title('Absorption Loss')

xlabel('Frequency (Hz)')

ylabel('Loss (dB)')

subplot(2,2,2)

semilogx(f,R,'LineWidth',2)

grid on

title('Reflection Loss')

xlabel('Frequency (Hz)')

ylabel('Loss (dB)')

subplot(2,2,3)

semilogx(f,SE,'LineWidth',2)

grid on

title('Shielding Effectiveness')

xlabel('Frequency (Hz)')

ylabel('SE (dB)')

subplot(2,2,4)

semilogx(f,Measured_Emission,...

'LineWidth',2)

hold on

semilogx(f,EMC_Limit,...

'r--','LineWidth',2)

grid on

legend('Measured EMI',...

'EMC Limit')

title('EMC Compliance Analysis')

xlabel('Frequency (Hz)')

ylabel('Emission Level (dBμV/m)')

OUTPUT: