9a.Simulation of Ground Wave and Sky Wave Propagation using MATLAB

OVERALL MATLAB CODE:

clc;

clear;

close all;

%% Objective 1: Ground Wave Propagation

Pt = 10; % Transmitted Power (kW)

d = 1:1:500; % Distance (km)

E_ground = (300*sqrt(Pt))./d;

%% Objective 2 : Sky Wave Propagation

h = 250; % Ionosphere Height (km)

theta = 60; % Angle of Incidence (degrees)

fc = 5; % Critical Frequency (MHz)

Skip_Distance = 2*h*tand(theta);

MUF = fc*sec(deg2rad(theta));

%% Objective 3 : Comparative Analysis

Sky_Range = Skip_Distance*ones(size(d));

%% Display Results

fprintf('\nGROUND WAVE AND SKY WAVE ANALYSIS\n');

fprintf('---------------------------------\n');

fprintf('Transmitted Power = %.2f kW\n',Pt);

fprintf('Ionospheric Height = %.2f km\n',h);

fprintf('Skip Distance = %.2f km\n',Skip_Distance);

fprintf('Maximum Usable Frequency (MUF) = %.2f MHz\n',MUF);

%% Plot Results

figure('Name',...

'Ground Wave and Sky Wave Propagation',...

'NumberTitle','off');

subplot(2,2,1)

plot(d,E_ground,'LineWidth',2)

grid on

xlabel('Distance (km)')

ylabel('Field Strength (mV/m)')

title('Ground Wave Propagation')

subplot(2,2,2)

bar(MUF)

grid on

title('Maximum Usable Frequency')

ylabel('Frequency (MHz)')

set(gca,'XTickLabel',{'MUF'})

subplot(2,2,3)

bar(Skip_Distance)

grid on

title('Skip Distance')

ylabel('Distance (km)')

set(gca,'XTickLabel',{'Sky Wave'})

subplot(2,2,4)

plot(d,E)

OUTPUT: